# ENERGY AND MATERIAL BALANCE

## OVERALL MATERIAL BALANCE:

ST-2

ST-1

ST-3

PROCESS

ST-4

ST-5

IN

| COMP | ST-1 FLUE GAS |  | ST-3 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/h | Mol% | Kg/h |
| N2 | 68.08 | 218.20. | - | - |
| O2 | 5 | 16.0255 | - | - |
| H2O | 12 | 38.46 | 98 | 4083.33 |
| CO2 | 13.5 | 43.36 | - | - |
| Ar | 2 | 6.4102. | - |  |
| LIZARDITE | - | - | 2 | 83.33 |

OUT

|  | ST-2 |  | ST-4 |  | ST-5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mol% | Kg/h | Mol% | Kg/h | Mol% | Kg/h |
| N2 | 93.16 | 217.69 | - | - | - | - |
| O2 | 3.3 | 7.71 | - | - | - | - |
| H2O | 0.5 | 1.168 | 100 | 4120.71 | 1 | 1.33 |
| CO2 | 1.84 | 4.299 | - | - | - | - |
| Ar | 1.2 | 2.8 | - | - | - | - |
| Nesqhionite | - | - | - | - | 99 | 131.43 |

CAPACITY

1 kg of CO2 requires 2 kg of Lizardite. So, 1 ton/day= 1000/24 kg/hr = 41.667kg/hr.

So, from the above statement it is clear that 41.667kg/hr of CO2 will require 83.33kg/hr of Lizardite.

So, total inlet flowrate of ST-3 = 83.33/0.02= 4166.7 kg/ hr of feed. From the above statement we know that

0.13*ST-1 = 41.667 kg/hr of CO2

ST-1 = 320.51 kg/hr.

N2 Balance

0.68*320.51 = 0.9316* ST-2

ST-2 = 233.68 kg/hr

H2O Balance

0.12*320.51 + 0.98*4166.5 = 0.005*233.68 + ST-3

ST-4 = 4120.41 kg/hr

ST-1 + ST-3 = ST-4 + ST-2 + ST-5

ST-5 = 132.76 kg/hr

## EQUIPMENT MATERIAL BALANCE:

### MATERIAL ON ABSORBER

Temp=8°C

Pressure=1.01325 bar

Material in:

| COMP | ST- 1 |  | ST-6 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/h | Mol% | Kg/h |
| N2 | 68.08 | 218.20. | - | - |
| O2 | 5 | 16.0255 | - | - |
| H2O | 12 | 38.46 | 64.64 | 67.206 |
| CO2 | 13.5 | 43.36 | 8.77 | 9.11 |
| Ar | 2 | 6.4102. | - | - |
| MEA | - | - | 26.6 | 27.65 |

Material out:

| COMP | ST-2 |  | ST-7 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/hr | Mol% | Kg/hr |
| N2 | 79.39 | 185.86 | - | - |
| O2 | 6.63 | 6.63 | - | - |
| H2O | 1 | 1 | 54.58 | 103.94 |
| CO2 | 15.74 | 36.86 | 22.57 | 42.98 |
| Ar | 0.956 | 2.2386 | - | - |
| MEA | - | - | 22.58 | 43.001 |

ST-1 + ST-6 = ST-2 + ST-7

320.51 - 233.68 = ST-7 - ST-6

H2O Balance

0.12*320.15 + 0.6464* ST-6 = 0.005*233.68 + 0.5485*(86.47+ ST-6)

ST-6 = 103.97 kg/hr

ST-7 = 190.44 kg/hr

### MATERIAL BALANCE ACROSS STRIPPER:

Material in:

| COMP | ST-7 |  |
| --- | --- | --- |
|  | Mol% | Kg/hr |
| H2O | 54.58 | 103.94 |
| CO2 | 22.57 | 42.98 |
| MEA | 22.58 | 43.001 |

Material out:

| COMP | ST-6 |  | ST-8 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/hr | Mol% | Kg/hr |
| H2O | 64.64 | 67.206 | 1 | 0.8657 |
| CO2 | 8.77 | 9.11 | 99 | 85.7043 |
| MEA | 26.6 | 27.65 | - | - |

ST-7 = ST-6 + ST-8

ST-8 = 86.57 kg/hr

### MATERIAL BALANCE ACROSS REACTOR:

Material in:

| COMP | ST-3 |  | ST-8 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/hr | Mol% | Kg/hr |
| H2O | 98 | 4083.366 | 1 | 0.8657 |
| CO2 | - | - | 99 | 85.7043 |
| LIZARDITE | 2 | 83.334 | - | - |

Material out:

| COMP | ST-9 |  |
| --- | --- | --- |
|  | Mol% | Kg/hr |
| LIZARDITE | 1 | 42.5327 |
| CO3 -2 | 0.057 | 2.24 |
| Mg+2 | 0.26 | 11.058 |
| SiO2 | 0.43 | 18.28 |
| H2O | 98.253 | 4178.98 |

ST-3 + ST-8 = ST-9

ST-9 = 4253.27 kg/hr

Molar mass of Lizardite = 277.11 g/mole

No. of moles = 83334/277.11 = 300.72 mole/hr

Fa = Fa0(1-Xa)

Where Xa at 45 C, 5 bar, 600 Pa in one hour is 0.5

Fa = 300.725(1-0.5) = 150.36 moles/hr

HCO3-2 Balance

Fb = Fb0 – Fa0*Xa*b/a

Mg3Si2O5(OH)4 + 6H+                                                             3Mg+2 + 2SiO2 5H2O

Fb0 = CO2 in feed/MW of CO2 = 4144/44 = 941.818 mol/hr

Fb = 941.818 – (300.725) *(0.5) *6

Fb = 39.643

Mg+2 Balance

Fc = Fc0 -Fa0*Xb*c/a

Fc = (300.725) *(0.5) *(3) = 451.0875 moles/hr

O2 Balance

Fd = Fd0 -Fa0*Xa*d/a

Fd = (300.725) * (0.5) * (2)

Fd = 300.725 moles/hr

Product stream

- Lizardite = 150.36 * 277.11 = 42.52 kg/hr

- CO3-2 = 39.643 * 60.009 = 2.42 kg/hr

- Mg+2 = 451.087 * 24.305 = 11.15 kg/hr

- SiO2 = 300.725 * 60 = 18.283 kg/hr

- H2O (out) = 4178.96 kg/hr

Total = 4253.23 kg/hr

Composition of the product stream

- Lizardite = 42.52/4253.23 = 0.01

- CO3-2 = 2.42/4253.23 = 0.00051

- Mg+2 = 11.15/4253.23 = 0.0026

- SiO2 = 18.283/4253.23 = 0.0043

- H2O = 4178.96/4253.23 = 0.9825

### MATERIAL BALANCE ACROSS FILTER 1:

Material in:

| COMP | ST-9 |  |
| --- | --- | --- |
|  | Mol% | Kg/hr |
| LIZARDITE | 1 | 42.5327 |
| CO3 -2 | 0.057 | 2.24 |
| Mg+2 | 0.26 | 11.058 |
| SiO2 | 0.43 | 18.28 |
| H2O | 98.253 | 4178.98 |

Material out:

| COMP | ST-10 |  | ST-11 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/hr | Mol% | Kg/hr |
| SiO2 | 0.5 | 10.6352 | - | - |
| LIZARDITE | 2 | 42.54 | - | - |
| H2O | 97.5 | 2073.864 | 99.51 | 2116.23 |
| Mg+2 | - | - | 0.09 | 1.91 |
| CO3-2 | - | - | 0.4 | 8.5 |

ST-9 = ST-10 +  ST-11

0.97* ST-10 + 0.99* ST-11 = 4178.83

0.975*(4253.27 - ST-11) + 0.99* ST-11 = 4178.83

ST-11 = 2126.66 kg/hr

ST-10 = 2127.04 kg/hr

MATERIAL BALANCE ACROSS PRECIPITAOR

Mg+2 + CO3-2 	MgCO3 (MAGNESITE)

MgCO3                		MgCO3.3H2O

Material in:

| COMP | ST-11 |  | ST-8 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/hr | Mol% | Kg/hr |
| CO2 | - | - | 99 | 85.7043 |
| H2O | 99.51 | 2116.23 | 1 | 0.8657 |
| Mg+2 | 0.09 | 1.91 | - | - |
| CO3-2 | 0.4 | 8.5 | - | - |

Material out:

| COMP | ST-12 |  |
| --- | --- | --- |
|  | Mol% | Kg/hr |
| MgCO3.3H2O | 0.43 | 9.51 |
| H2O | 99.56 | 2203.47 |

ST-11 + ST-8 = ST-12

ST-12 = 2213.21 kg/hr

Molecular mass of Mg+2 = 24.305 g/mol

No. of moles = 11058/24.305 = 454.96

Mg+2 Balance

Fa0 = 454.96

Fa = Fa0(1-Xa) = 454.96(1-0.5) = 227.48 moles/hr

CO3-2 Balance

Fb = Fb0(1-Xa)*b/a

Fb = 40.39 + (227.48) (0.5)

Fb = 154.13 moles/hr

MgCO3 Balance

Fc = Fc0-Fa0(Xa)

Fc = (227.48) (0.5)

Fc = 113.74 moles/hr

Product stream

MgCO3 = 84.313 * 113.74 = 9589.76/1000 = 9.58 kg/hr

9.58/2213.21 = 0.0043

1- 0.0043 = 0.9956

### MATERIAL BALANCE ACROSS FILTER 2:

ST-12 = ST-15 + ST-13

ST-13 = 2080.09 kg/hr

ST-15 = 133.12 kg/hr

Material in:

| COMP | ST-12 |  |
| --- | --- | --- |
|  | Mol% | Kg/hr |
| MgCO3.3H2O | 0.43 | 9.51 |
| H2O | 99.56 | 2203.47 |

Material out:

| COMP | ST-13 |  | ST-15 |  |
| --- | --- | --- | --- | --- |
|  | Mol% | Kg/hr | Mol% | Kg/hr |
| MgCO3.3H2O | 0.2 | 4.16 | 99 | 131.788 |
| H2O | 99.56 | 2070.93 | 1 | 1.33 |

## OVERALL ENERGY BALANCE:

Overall Energy Balance:

Energy balance accounts for energy being transferred to or from the each equipment and is done here by taking into account the law of conservation of energy that is defined as bellow.

Law of conservation of energy:

It states that:

[Rate of energy going into the system] - [Rate of energy going out of the system] + [Rate of energy generation within the system] - [Rate of energy consumption within the system] = [Rate of energy accumulation in the system]

Now the energy balance of each equipment is given bellow

ST-4

WASH WATER

ST-3

TREATED FLUE GAS

ST-2

COMPRES CO2

PROCESS

ST-1

FLUE GAS

ST-6

CONDENS WATER

ST-5

MAKEUP AMMONIA SOLUTION

| STREAM NO | ∆T | MASS FLOW | AVERAGE CP | AVERAGE DENSITY | m*cp*∆T | m*∆P/ϸ | Q |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (k) | Kg/h | Kj/kg.k | Kg/m3 | KW | KW | KW |
| 1 | 393 | 320.51 | 1.39 | 0 | 48.63 | - | 48.63 |
| 2 | 323 | 233.94 | 1.0347 | 0 | 21.71 | 0.13 | 21.71 |
| 3 | 298 | 4166.7 | 4.068 | 0 | 1403.09 | - | 1403.09 |
| 4 | 313 | 4120.71 | 4.0737 | 0 | 1459.49 | - | 1459.49 |
| 5 | 313 | 132.56 | 1.5 | 0 | 17.28 | - | 17.28 |

Stream in:		81.28-0.77-.013 = 80.49 KW

Stream out		-8.79-27.36-13.85 = -50.03 KW

## EQUIPMENT ENERGY BALANCE

### ENERGY BALANCE ACROSS DIRECT CONTACT COOLER:

| STREAM | TEMPERATURE | AVERAGE CP | FLOW RATE | Q |
| --- | --- | --- | --- | --- |
|  | °C | Kj/kg.k | Kg/h | KW |
| FLUE GAS INTO DCC-1 | 50 | 1.49 | 7872.94 | 81.46 |
| FLUE GAS FROM DCC-1 | 30 | 1.49 | 7872.94 | 16.29 |

### ENERGY BALANCE ACROSS HEAT EXCHANGER (BEFORE ABSORBER)

| STREAM | TEMPERATURE | AVERAGE CP | FLOW RATE | Q |
| --- | --- | --- | --- | --- |
|  | °C | Kj/kg.k | Kg/h | KW |
| FLUE GAS INTO CHILLER-1 | 30 | 1.49 | 7872.94 | 16.29 |
| FLUE GAS FROM CHILLER-1 | 8 | 1.49 | 7872.94 | -55.39 |

### ABSORBER ENERGY BALANCE:

HEAT OF REACTION:

∆Hrxn= ∑(νi*∆Hf,i)products-∑(νi*∆Hf,i)reactants

For 1st:

1(-940.772) - (-285.8-393.5-2*46.2) = -169.1 kj/mole

For 2nd:

1(-853.385) - (-285.8-393.5-46.2) = -127.885 kj/mole

For 3rd:

2(-853.385) - (-940.772-393.5-285.8) = -86.69 kj/mole

For 4th:

1(151.8) - (-393.5-2*46.2) = 637.7 kj/mole

| COMPONENTS | HEAT OF FORMATION |
| --- | --- |
| CO2 | -393.5 |
| H2O | -285.8 |
| NH3 | -46.2 |
| NH4HCO3 | -853.385 |
| (NH4)2CO3 | -940.772 |
| NH4CO2NH2 | 151.8 |

| STREAM | TEMPERATURE | AVERAGE CP | FLOW RATE | Q |
| --- | --- | --- | --- | --- |
|  | °C | Kj/kg.k | Kg/h | KW |
| FLUE GAS INTO ABSORBER | 8 | 1.49 | 7872.94 | 81.46 |
| FLUE GAS FROM TOP OF THE ABSORBER | 9 | 1.55 | 6416.37 | -16.29 |
| LEAN SOLUTION INTO ABSORBER | 8 | 3.09 | 3732.334 | -54.46 |
| RICH SOLUTION DISCHARGE FROM THE ABSORBER | 50 | 3.28 | 5191.16 | 118.24 |

### ENERGY BALANCE ACROSS STRIPPER:

| STREAM | TEMPERATURE | AVERAGE CP | FLOW RATE | Q |
| --- | --- | --- | --- | --- |
|  | °C | Kj/kg.k | Kg/h | KW |
| RICH SOLUTION FROM STRIPPER | 120 | 3.025 | 136.098 | 44.94 |
| LEAN SOLUTION FROM ABSORBER | 70 | 2.694 | 222.668 | 57.15 |
| COMPRESSOD CO2 | 120 | 0.939 | 86.57 | 8.87 |

### INTERMEDIATE HEAT EXCHANGER ENERGY BALANCE:

| STREAM | TEMPERATURE | AVERAGE CP | FLOW RATE | Q |
| --- | --- | --- | --- | --- |
|  | °C | Kj/kg.k | Kg/h | KW |
| LEAN SOLUTION FORM STRIPPER BEFORE HE-1 | 110 | 3.09 | 3732.334 | 272.30 |
| LEAN SOLUTION INTO ABSORBER AFTER HE-1 | 70 | 3.09 | 3732.334 | 144.161 |
| RICH SOLUTION FROM ABSORBER BEFORE HE-1 | 50 | 3.28 | 5191.16 | 118.24 |
| RICH SOLUTION INTO STRIPPER AFTER HE-1 | 90 | 3.28 | 5191.16 | 307.43 |